You can squeeze surprisingly high PSRAM throughput out of the OMGS3’s QSPI PSRAM, but you have to tune ESP-IDF in a way that matches the actual hardware on the board. The OMGS3 uses:

- QSPI PSRAM (4-bit, STR mode)

- QSPI Flash

- ESP32-S3 memory controller

That combination has a very specific set of knobs you can turn. Below is the clean, practical tuning guide I’d use if I were optimizing your wand’s ML + fusion pipeline.

🚀 1. Enable the Fastest PSRAM Mode Supported by OMGS3

In menuconfig:

Component config → ESP32S3-specific → Support for external, SPI-connected RAM

Set:

- PSRAM clock → 80 MHz

- PSRAM mode → QSPI (4-bit)

- PSRAM speed → 80 MHz STR

The OMGS3’s PSRAM is stable at 80 MHz STR.
Do not enable OPI or DTR modes — the hardware doesn’t support them.

⚙️ 2. Enable “Cache for External RAM” (Critical for Throughput)

This is the single biggest performance win.

In menuconfig:

Component config → ESP System Settings → External RAM

Enable:

- “Cache external RAM”

- “Allow external memory as heap”

- “Allow external memory for DMA” (if you use DMA-heavy workloads)

This allows the CPU to use the instruction/data cache when accessing PSRAM, which boosts throughput dramatically.

Without caching, PSRAM is ~8–12 MB/s.
With caching, you can hit ~40–60 MB/s depending on access pattern.

🧠 3. Use 32-bit Aligned Accesses

The ESP32-S3 memory controller is happiest when:

- reads/writes are 32-bit aligned

- buffers are word-aligned

- you avoid byte-wise operations in tight loops

For ML inference or gesture pipelines:

- allocate buffers with heap_caps_malloc(size, MALLOC_CAP_SPIRAM | MALLOC_CAP_32BIT)

- ensure your feature vectors and ring buffers are 4-byte aligned

This alone can double throughput for some workloads.

🧵 4. Pin High-Bandwidth Tasks to Core 1

Core 0 handles Wi-Fi/BLE/RTOS housekeeping.
Core 1 is your “compute core.”

For maximum PSRAM throughput:

- run your Madgwick fusion,

- TinyML inference,

- gesture feature extraction,

- large buffer operations

on Core 1.

Example:

xTaskCreatePinnedToCore(task_fn, "fusion", 4096, NULL, 5, NULL, 1);

This prevents cache thrash and PSRAM stalls caused by Wi-Fi/BLE interrupts.

🧹 5. Avoid Frequent malloc/free in PSRAM

PSRAM allocation is slower and more fragmented than internal RAM.

Best practice:

- allocate large buffers once at startup

- reuse them

- avoid dynamic allocation in your gesture loop

This keeps the PSRAM heap contiguous and avoids slow path allocators.

🔄 6. Use Burst-Friendly Access Patterns

The PSRAM controller performs best when you:

- read/write in large contiguous blocks

- avoid random access

- avoid byte-at-a-time operations

- use memcpy for bulk moves (it’s optimized)

For your wand:

- store IMU windows as contiguous arrays

- process them sequentially

- avoid scatter-gather patterns

🧪 7. Enable “SPIRAM Speed Optimization” (If Available)

In some ESP-IDF versions:

Component config → ESP32S3-specific → PSRAM

Enable:

- “Optimize SPIRAM access speed”

This tweaks timing margins and cache behavior.

🧰 8. Use the Right Heap Caps for Your Use Case

For ML and fusion:

- MALLOC_CAP_SPIRAM → general PSRAM

- MALLOC_CAP_8BIT → byte-addressable

- MALLOC_CAP_32BIT → fastest access

- MALLOC_CAP_DMA → if using I2S/camera/etc.

Example:

float *window = heap_caps_malloc(1024 * sizeof(float),

MALLOC_CAP_SPIRAM | MALLOC_CAP_32BIT);

📈 9. Benchmark Your Throughput

ESP-IDF includes a PSRAM benchmark:

idf.py menuconfig → Component config → ESP32S3 → Test external RAM

Or write your own:

uint32_t start = esp_timer_get_time();

memcpy(dst, src, 1024*1024);

uint32_t end = esp_timer_get_time();

printf("MB/s = %f\n", 1.0 / ((end-start)/1e6));

This lets you confirm your tuning is actually working.

🧙 10. What You Can Expect on the OMGS3

With all tuning applied:

| Mode | Throughput |
| --- | --- |
| Uncached PSRAM | ~8–12 MB/s |
| Cached PSRAM | 40–60 MB/s |
| Internal RAM | 200–300 MB/s |

For your wand:

- Madgwick fusion is tiny → runs in internal RAM

- TinyML inference uses PSRAM buffers → benefits massively from caching

- Gesture windows (e.g., 200–500 samples) → perfect for cached PSRAM

If you want, I can also help you:

- build a PSRAM-optimized gesture pipeline

- design a zero-copy IMU ring buffer

- tune your TinyML model for PSRAM locality

- write a PSRAM benchmark task for your OMGS3

Just tell me where you want to go next.