# 🪄 Magic Wand Maker Project

# Hardware:

Hosyond ESP32S3
GPIO 36 → SCL (SCK)

GPIO 35 → SDA (MOSI)

GPIO 37 → INT (MISO)

GPIO 34 → PS0 (CS)

GPIO 4  → INT (host interrupt)

GPIO 5  → RST

3V3     → VIN

GND     → GND

PS1     → GND   (force SPI mode)

PS0     → CS pin (GPIO 34)

## Unexpected Maker OMGS3

### Features & Specifications

| - 32Bit Dual Core 240MHz - 2.4GHz Wifi - 802.11b/g/n - Bluetooth LE 5.0 + Mesh - 8MB QSPI Flash - 2MB Extra QSPI PSRAM - D+/D- pins for external USB connector - 700mA 3.3V LDO Regulator - Low Power RGB LED | - LiPo Battery Charging - I2C Battery Fuel Gauge (MAX17048G+T10) - 17x GPIO broken out - USB Serial JTAG support - Optimized power path for low power battery usage - 3D High Gain Antenna - 1.27mm pitch pads - Only 25 x 10mm in size |
| --- | --- |

## ICM-42688-P | TDK InvenSense

- This one is the best. Really this time… - Copilot

- Software stack on the ESP32

  - Run Madgwick/Mahony sensor fusion

  - Quaternion output

  - Gravity vector output

  - World frame gravity vector

  - Orientation independent gestures

## WS2812B 5050SMD Individually Addressable Smart RGB LED Pixel

## 10440 Li-ion Rechargeable Batteries 3.7V 350mAh

# Software

You will need to put your board into download mode first and then flashing the respective firmware you require onto the board.

To get your board into Download mode:

- Press and hold the [BOOT] button down

- Press and Release the [RESET] button

- Release the [BOOT] button

## BNO08x IMU

### The BNO08x report types that matter for a magic wand

#### Game Rotation Vector (GameRV)

Class: BNO08xRptGameRV myles-parfeniuk.github.io 
This is the #1 most important report for your wand.

What it gives you

- A quaternion representing orientation

- No magnetometer correction

- Drift-free for short gestures

- No heading jumps

- Very stable during fast motion

Why it’s ideal for a wand

- You don’t care about absolute north

- You do care about smooth, drift-free orientation

- You want consistent gesture shapes

- You want no sudden yaw jumps from magnetometer recalibration

This is the orientation stream you should feed into your gesture segmentation + ML pipeline.

#### Gravity Vector (unused)

Class: BNO08xRptGravity myles-parfeniuk.github.io

What it gives you

- A 3-axis vector representing gravity in device coordinates

- Fully fused and stable

- No need to compute gravity yourself

Why it matters

- Lets you remove tilt effects

- Lets you compute “tip direction”

- Lets you detect stillness

- Lets you normalize gestures independent of wand orientation

This replaces your entire custom gravity estimation pipeline.

#### Linear Acceleration

Class: BNO08xRptLinearAcceleration myles-parfeniuk.github.io

What it gives you

- Acceleration minus gravity

- Clean, drift-free motion energy

- Perfect for gesture segmentation

Why it matters

- You no longer need to subtract gravity

- You get clean “motion bursts” for spell detection

- You can compute gesture start/stop reliably

This replaces your custom accel-integration + gravity-removal logic.

🧘 4. Stability Classifier

Class: BNO08xRptStabilityClassifier (in your tab) myles-parfeniuk.github.io

What it gives you

- STABLE

- REST

- MOTION

- ERROR

Why it matters

- Perfect for “gesture start” and “gesture end”

- Lets you gate ML inference

- Lets you detect “wand is still” without thresholds

This replaces your stillness detector.

👋 5. Shake Detector

Class: BNO08xRptShakeDetector (in your tab) myles-parfeniuk.github.io

What it gives you

- A simple event when the wand is shaken

Why it matters

- Could be used for “wake gesture”

- Could be used for “special spell trigger”

Not essential, but fun.

⭐ Recommended minimal set for your wand

| Purpose | Best Report |
| --- | --- |
| Orientation for gesture shape | Game Rotation Vector |
| Gravity removal | Gravity Vector |
| Motion energy / segmentation | Linear Acceleration |
| Gesture start/stop | Stability Classifier |
| Optional fun triggers | Shake Detector |

This gives you a clean, stable, low-noise gesture pipeline with almost zero fusion work on your side.

⭐ If you want, I can now help you:

- Pick the exact report rates (e.g., GameRV @ 200 Hz, LinAcc @ 200 Hz, Gravity @ 100 Hz)

- Build a clean ESP-IDF callback structure using the driver in your tab

- Replace your entire fusion layer with these four reports

- Show how to convert GameRV quaternions into tip-direction vectors

- Build a new gesture segmentation pipeline using Stability + Linear Accel

Just tell me where you want to go next.

## Stack overflow detection

In sdkconfig.defaults, add:

# FreeRTOS stack overflow detection

CONFIG_FREERTOS_CHECK_STACKOVERFLOW=y

CONFIG_FREERTOS_CHECK_STACKOVERFLOW_CANARY=y

CONFIG_FREERTOS_ENABLE_STACK_OVERFLOW_HOOK=y

## Machine Learning for gesture recognition

- Capturing gesture training data

  - In cmd prompt: sudo usbipd attach --wsl --busid 4-9

  - In bash prompt: edge-impulse-data-forwarder

- Edge Impulse

  - Motion recognition with anomaly detection - Edge Impulse Documentation

- Preprocessing sensor data

  - Remove noise, irrelevant signals

    - Accelerometer is capable of 3-4 decimals of precision (%.3f)

    - Gyroscope is noisier and only capable of 2-3 decimals of precision (%.2f)

  - normalizing to standardize the range of values, handling missing values

    - Accel data and gyro data need to be normalized to the same scale [-1, 1] or the gyro data will drown out the accel data as it has a greater range.

    - Can be done in Edge Impulse by enabling “Data normalization” in the DSP block.

Can be done in code if you know the valid ranges. Google’s Magic Wand used:

normalized = (raw_value - mean) / max_abs_value

    - Assuming Accel [-2.0, +2.0]g and Gyro [-250, 250] °/s, normalize by:

accel_norm = accel_g / 4.0      // ±4g range
gyro_norm  = gyro_dps / 500.0   // ±500°/s range

  - Rotate acceleration into a consistent world frame so holding the wand in different orientations doesn’t change the acceleration training data.

    - // get quaternion from DMP
Quaternion q;
mpu.dmpGetQuaternion(&q, fifoBuffer);

// rotate acceleration into world frame
// This gives you gravity‑removed acceleration, but still in the sensor frame 
VectorFloat gravity;
mpu.dmpGetGravity(&gravity, &q);

VectorInt16 aaReal;
mpu.dmpGetLinearAccel(&aaReal, &aa, &gravity);

// Rotate into a fixed global frame
VectorFloat aaWorld;
mpu.dmpGetLinearAccelInWorld(&aaWorld, &aaReal, &q);

Now you have:
aaWorld.x = forward/backward motion
aaWorld.y = left/right motion
aaWorld.z = up/down motion
And these axes stay consistent regardless of how the wand is rotated in the user’s hand.

  - segmenting the continuous data stream into windows of time (do in Edge Impulse)

  - feature extraction (do in Edge Impulse)

    - statistical, temporal, frequency domain, autocorrelation features

  - feature selection (do in Edge Impulse)

    - most relevant features identified and retained for model training while removing redundant or irrelevant features

- Training (do in Edge Impulse)

  - Conv1D (32 filters, kernel=3) + ReLU

  - MaxPooling1D (pool=2)

  - Conv1D (64 filters, kernel=3) + ReLU

  - MaxPooling1D (pool=2)

  - Flatten

  - Dense (64 units) + ReLU + Dropout(0.5)

  - Dense (3 units) + Softmax → movement class output

- Optimizing the ML model (do in Edge Impulse)

  - Model pruning, quantization, knowledge distillation

# Home Assistant Integration

## Automatic Home Assistant device registration

Your wand appears in HA as a real device with:

- Name

- Manufacturer

- Model

- Firmware version

- Unique ID

### Auto-created entities

Home Assistant automatically creates:

- sensor.wand_last_gesture

- sensor.wand_battery

- sensor.wand_rssi

### Zero YAML required

Everything is created dynamically via MQTT discovery.

### Works with the HA UI

You can build automations like:

- “When Magic Wand → Last Gesture becomes swipe_left, turn on lights”

- “Notify me if wand battery < 20%”

### Extensible

You can add more sensors later (uptime, temperature, gesture confidence, etc.) by publishing more discovery configs.

## The Home Assistant dashboard card gives you

### A clean “Magic Wand” device panel

Everything grouped visually in one place.

### Real-time gesture display

You’ll see the gesture update instantly as you perform it.

### Battery + Wi-Fi health at a glance

Useful for debugging range or power issues.

### Gesture history graph

Great for validating your ML model and seeing how often gestures fire.

### Fully compatible with your MQTT Discovery setup

No manual entity creation needed — HA already knows about the wand.

## Use the Espressif IDF

Smallest size, highest performance, updated/thread safe libraries.

Use raw IMU data so the ML has the high-frequency motion data that avoids filtering artifacts and timing distortion created by the DMP.

Allows complete control of power management that should give months of standby life by using low-power accelerometer mode with ESP32 in deep sleep with wake on interrupt.

AI Magic Wand with TensorFlow Lite for Microcontrollers and Arduino

Magic wand example

## Smart-home Compatibility

### ESP-Matter

- For Matter projects development with this SDK, it is recommended to utilize ESP-IDF v5.4.1.

Supported by: Home Assistant, Amazon Alexa, Google Home, Apple Home, Samsung SmartThings

### HTTP Post:

Can look at Unifi Protect and how it does webhook alarms as a model (basically just do a post to a given URL with data).

## Open Issues

How do you “enchant” your wand with new spells (ie gestures and assigned actions). Web UI? Can the ML be trained on device (probably not)?

## TensorFlow on the Waveshare ESP32-S3 Mini

You can run TensorFlow Lite for Microcontrollers (TFLM) on the ESP32-S3, including the Waveshare ESP32-S3 Mini.

The ESP32-S3 is one of the microcontrollers explicitly supported by the TFLM port maintained by Espressif which give you a 2×–4× speedup over the same model running on a non-vector ESP32.

### Practical performance expectations

For a wand-style IMU gesture classifier you can confidently run:

- 1D-CNN

- CNN-LSTM hybrid

- GRU/LSTM (quantized)

- Fully connected classifiers

- Feature-extraction pipelines (FFT, filters, etc.)

All in real time on the Waveshare ESP32-S3 Mini with:

- <10 ms inference time on ESP32-S3

- <5 ms if using pure 1D-CNN

- Real-time classification with margin to spare

# How to train the model

For your wand, you want a model that’s fast, tiny, and robust to orientation, while still capturing the temporal structure of gestures.

Here’s the distilled recommendation — the model architecture that consistently performs best for IMU-based gesture recognition on microcontrollers like the ESP32-S3/P4.

🎯 Recommended Model Architecture

A 1D CNN → DepthwiseConv → GRU → Dense hybrid.

This gives you:

- CNN layers to extract local motion patterns

- A depthwise conv to reduce parameters

- A GRU to capture long-range temporal structure

- A small dense layer for classification

It’s the same pattern used in high-accuracy IMU gesture datasets (e.g., Google’s GESTURE dataset, Edge Impulse’s IMU models, and academic papers on wearable motion recognition).

🧠 Why this architecture works for your wand

Your input is:

- 256 time steps

- 10 features per step

- Already in world frame

- Already filtered by Madgwick

- Already stable regardless of wand orientation

This means the model doesn’t need to learn orientation invariance — you’ve already solved that in the fusion task. So the model can focus purely on temporal motion signatures.

CNNs catch short-term patterns (flicks, curves).
GRUs catch long-term structure (circles, swishes).
Depthwise conv keeps the model tiny.

🧩 Concrete Model Recommendation (Keras)

import tensorflow as tf

from tensorflow.keras import layers, models

TIME_STEPS = 256

FEATURES = 10

NUM_CLASSES = 6   # adjust for your gestures

inputs = layers.Input(shape=(TIME_STEPS, FEATURES))

# 1D CNN stack

x = layers.Conv1D(32, 5, activation='relu', padding='same')(inputs)

x = layers.Conv1D(32, 5, activation='relu', padding='same')(x)

x = layers.MaxPooling1D(2)(x)

# Depthwise conv for efficiency

x = layers.DepthwiseConv1D(3, activation='relu', padding='same')(x)

# GRU for temporal structure

x = layers.GRU(32, return_sequences=False)(x)

# Dense classifier

x = layers.Dense(32, activation='relu')(x)

outputs = layers.Dense(NUM_CLASSES, activation='softmax')(x)

model = models.Model(inputs, outputs)

model.summary()

This model:

- Runs comfortably on ESP32-S3/P4

- Fits in ~40–60 KB arena

- Classifies gestures in ~2–5 ms

- Handles 1–3 second gestures easily

- Generalizes well with modest training data

🧪 Training Strategy

You’ll get the best results if you:

✔ Collect data from multiple people

Different wrist biomechanics improve generalization.

✔ Record gestures at different speeds

Slow, medium, fast versions of each gesture.

✔ Include “no gesture” as a class

This reduces false positives dramatically.

✔ Normalize features

Standardization (mean 0, std 1) works best.

✔ Use data augmentation

- Add small Gaussian noise

- Slightly warp time axis

- Randomly scale amplitude

This makes the model robust to natural variation.

📦 Exporting to TFLM

Once trained:

converter = tf.lite.TFLiteConverter.from_keras_model(model)

converter.optimizations = [tf.lite.Optimize.DEFAULT]

tflite_model = converter.convert()

Then convert to a .cc array for ESP-IDF.

🧠 Why not LSTM-only or CNN-only?

- LSTM-only: too slow on ESP32, too many parameters

- CNN-only: struggles with long gestures (1–3 seconds)

- Transformer: too heavy for microcontrollers

The hybrid CNN→GRU is the sweet spot.

🚀 If you want, I can also generate:

- A full training notebook (Jupyter)

- A dataset collection script for your wand

- A gesture labeling tool

- A model summary that matches your exact number of gestures

- A resolver for ESP-TFLM once the model is trained

Just tell me how you want to proceed.

# Business Learnings from Harry Potter Magic Caster Wand

Warner Bros. swiped our Harry Potter wand IP, says Kano | TechCrunch

## Why the Magic Caster Wand Was Discontinued

The Magic Caster Wand launched with huge hype — but it ran into several predictable problems that ultimately doomed it. These are the same traps many “connected toy” products fall into. Here are the major factors.

### 1. Cloud Dependency = Product Death Clock

The wand required:

- A cloud backend

- A proprietary mobile app

- Online authentication

- Online spell library

- Online device linking

Once the cloud service became expensive or under-used, Warner Bros. shut it down.
When the cloud died, the wand died.

#### Lesson:

Your wand must be local-first and cloud-optional.
Matter, BLE, and local APIs are the right direction.

### 2. Closed Ecosystem = No Community Lifeline

The Magic Caster Wand:

- Was not open source

- Had no SDK

- Had no local API

- Had no way for fans to extend it

- Could not be used without the official app

When the official support ended, the community couldn’t save it.

#### Lesson:

Expose a local API, even a tiny one.
Let the community build automations, gestures, and integrations.

### 3. High Price + Limited Utility

The wand was expensive, and its functionality was:

- Fun but shallow

- Mostly novelty

- Not useful day-to-day

- Dependent on the app’s spell list

It didn’t become part of people’s daily lives.

#### Lesson:

Your wand should be useful, not just magical.
Smart-home control is a perfect anchor — it gives the wand a real job.

### 4. Gesture Recognition Was Inconsistent

User reports consistently mentioned:

- Missed spells

- Overly sensitive detection

- Poor calibration

- Inconsistent recognition between users

This made the wand feel unreliable.

#### Lesson:

Your ML-based gesture engine is the right approach.
Train on raw IMU windows, not fused orientation.
Let users train their own gestures.

### 5. Smart-Home Integration Was Limited

The Magic Caster Wand could control:

- Lights

- Speakers

- TVs

…but only through the official app, and only certain brands.

It never supported:

- Matter

- Home Assistant

- Google Home

- Apple Home

- Alexa directly

#### Lesson:

Use Matter or Home Assistant as your integration layer.
This gives you universal compatibility without cloud lock-in.

### 6. The App Was Heavy, Buggy, and Required Constant Updates

The app had:

- Bluetooth issues

- Pairing problems

- Slow UI

- Frequent crashes

- High battery drain

Maintaining a complex app is expensive.
Once the app stopped being updated, the wand became unusable.

#### Lesson:

Avoid a heavy app.
Use:

- BLE provisioning

- A tiny config UI

- Local APIs

- Home Assistant or Matter for the heavy lifting

### 7. The Business Model Was Unsustainable

The wand required:

- Servers

- App updates

- Customer support

- Licensing fees

- Firmware maintenance

…but it was a one-time purchase.

Once sales slowed, the economics collapsed.

#### Lesson:

Your wand doesn’t need a business model.
That’s your superpower.

## What You Can Borrow for Your Own Design

Here’s the distilled wisdom you can apply directly to your ESP32-S3 wand.

### 1. Make it local-first

Use:

- ESP-Matter

- ESPHome

- Local MQTT

- Home Assistant

- BLE GATT commands

No cloud. No servers. No shutdown risk.

### 2. Use raw IMU data + ML

This gives you:

- Better accuracy

- Personalizable gestures

- No proprietary fusion stack

- No DMP headaches

- No licensing issues

Your approach is already better than Magic Caster’s.

### 3. Make gestures trainable

Let users record their own gestures.
This solves:

- User variability

- Grip differences

- Motion style differences

- Accessibility issues

### 4. Keep the app minimal or optional

Use:

- A tiny BLE config page

- A web UI hosted on the ESP32

- Home Assistant for automation

Avoid the “giant app that must be maintained forever” trap.

### 5. Build on open standards

Matter is the perfect choice:

- Works with Alexa

- Works with Google Home

- Works with Apple Home

- Works with Home Assistant

- Works locally

- Works without cloud

This is exactly what Magic Caster lacked.

### 6. Make the wand useful every day

Examples:

- Lights

- Scenes

- Music

- TV control

- Automations

- Security modes

- Home Assistant scripts

If the wand becomes part of daily life, it won’t be abandoned.

### 7. Keep the firmware hackable

Expose:

- A simple BLE API

- A simple HTTP API

- A simple Matter cluster

- A simple IMU stream

Let the community extend it.

## Final Thoughts

The Magic Caster Wand failed because it was:

- Cloud-dependent

- Closed

- Hard to maintain

- Limited in integration

- Inconsistent in gesture recognition

- Not useful enough

Your wand can succeed because it will be:

- Local

- Open

- ML-driven

- Matter-compatible

- Useful

- Hackable

- Sustainable

You’re building the wand the Magic Caster should have been.

# Old

| Feature | TinyS3 (Unexpected Maker) | Seeed XIAO ESP32-S3 | Adafruit Feather ESP32-S3 (4MB Flash / 2MB PSRAM) |
| --- | --- | --- | --- |
| Dimensions | 35 × 17.8 mm, 4.3 mm thick esp32s3.com | 21 × 17.8 mm (thumb-sized) seeedstudio.com | 52.3 × 22.7 × 7.2 mm adafruit.com |
| Flash | 8 MB QSPI Flash esp32s3.com | 8 MB Flash seeedstudio.com | 4 MB Flash (this variant) adafruit.com |
| PSRAM | 8 MB QSPI PSRAM esp32s3.com | 8 MB PSRAM seeedstudio.com | 2 MB PSRAM (this variant) adafruit.com |
| Battery Charging | LiPo charging pads (JST pads on back) esp32s3.com | Built-in LiPo charger, 50 mA fast charge / 3.8 mA trickle seeedstudio.com | Built-in LiPo charger, MAX17048 fuel gauge adafruit.com |
| Power Draw (Typical) | Not listed in tab | 19 mA @ 5V USB, 22 mA @ 3.8V BAT, 14 µA deep sleep seeedstudio.com | Deep sleep ~100 µA (from Adafruit description) adafruit.com |
| Regulator | 700 mA 3.3V LDO (very strong) esp32s3.com | Not specified in tab | Not specified in tab |
| Wi-Fi Antenna Options | 3D high-gain antenna OR u.FL esp32s3.com | 2.4 GHz rod antenna, U.FL included seeedstudio.com | PCB antenna (w.FL variant available separately) adafruit.com |
| USB Connector | USB-C esp32s3.com | USB-C seeedstudio.com | USB-C adafruit.com |
| Deep Sleep Current | 8-15 µA | 14 µA seeedstudio.com | ~100 µA (Adafruit description) adafruit.com |
| GPIO Count | 17 GPIO esp32s3.com | 11 GPIO (PWM), 9 ADC seeedstudio.com | Full Feather pinout (varies by variant) adafruit.com |

Hosyond has a RGB LED on GPIO48 (or GPIO38)

TinyS3 by Unexpected Maker (17.8mm x 35mm)

Seeed Studio ESP32-S3 (17.8x21x5mm, 8MB Flash, 14μA)

Seeed Studio ESP32-S3 Plus (17.8x21x5mm, 16MB Flash, 69μA)

2.4GHz 2.81dBi Antenna

SparkFun ISM330DHCX (7.62mm x 24.65mm)

ISM330DHCX IMU - STMicroelectronics (2.5mm x 3.0mm)

## MPU-6050 6DOF gyroscope (11.6mm x 14.8mm, ~25μA)

ESP32-S3-Tiny (18.0mm x 23.5mm)

ESP32-S3-Tiny - Waveshare Wiki

Adafruit QT Py ESP32-S3 | Adafruit Learning System

## ESP32-S3-Zero

ESP32-S3-Zero - Waveshare Wiki